ATEK1601

Programming Supplement

2-18 GHz Sub-Octave USB-Controlled Filter Bank

Document revision 1.1  |  Python API version 1.1.0

This supplement contains the ATEK1601-specific programming information. It is used together with the ATEK RF Modules Programming Manual, which describes the connection, the command set, state reporting and the Python API common to all modules.

Software and example programs: https://github.com/atek-midas/atek-rf-modules

# 1  Module Identification

| Item | Value |
| --- | --- |
| Module name (*IDN? response) | ATEK1601 |
| RF chip | ATEK656N5 |
| Description | 2-18 GHz Sub-Octave USB-Controlled Filter Bank |
| States | 6 (state 0 … 5) |
| Python class | atek.ATEK1601 |

RF performance data is found in the module datasheet: https://atekmidas.com/products/modules/atek1601/

# 2  States

6-band sub-octave filter bank. Band n is selected with state n − 1. The front-panel buttons step through the same states.

6 bands. Pass bands:

| State | Band | Pass band (GHz) |
| --- | --- | --- |
| 0 | Band 1 | 1.9 – 3.5 |
| 1 | Band 2 | 2.8 – 5.4 |
| 2 | Band 3 | 4.5 – 9.1 |
| 3 | Band 4 | 7.1 – 12.3 |
| 4 | Band 5 | 9.9 – 15.3 |
| 5 | Band 6 | 12.5 – 18 |

# 3  Raw Command Example

| Direction | Line | Comment |
| --- | --- | --- |
| PC → Module | *IDN? | Identify the module |
| Module → PC | ATEK1601 |  |
| PC → Module | SET:3 | Select band 4 |
| Module → PC | STATE:3 | Confirmation |
| PC → Module | GET? | Read the current state |
| Module → PC | STATE:3 | Band 4, 7.1–12.3 GHz |

Command syntax and state reporting are described in Section 3 of the ATEK RF Modules Programming Manual.

# 4  Python API

## 4.1  Class atek.ATEK1601

atek.connect() returns an atek.ATEK1601 object when a ATEK1601 is connected. In addition to the common methods (ATEK RF Modules Programming Manual, Section 5.5) it provides:

| Method / attribute | Description |
| --- | --- |
| set_band(band, verify=True) | Selects band 1…6. Returns the band. |
| get_band() | Returns the active band (1…6). |
| get_band_range_mhz(band=None) | Returns (low, high) of a band in MHz; default is the active band. |
| set_frequency_mhz(mhz, verify=True) | Selects the band that best covers mhz (the band in which mhz is closest to the band centre). Returns the band. |
| num_bands | Number of bands (6). |
| BANDS_MHZ | Tuple of the (low, high) pass bands in MHz, index = state. |

Band numbers start at 1; state indexes start at 0. Frequencies are given in MHz.

## 4.2  Example program

import atek_rf_modules_scpi_api as atek

with atek.connect("COM5") as fb:               # returns an atek.ATEK1601 object

print("IDN    :", fb.identify())             # ATEK1601

fb.set_band(3)                                # band 3 (state 2)

print("Band   :", fb.get_band(), "->", fb.get_band_range_mhz(), "MHz")

band = fb.set_frequency_mhz(10000)            # best band for 10 GHz

print("10 GHz :", "band", band, "->", fb.get_band_range_mhz(), "MHz")

Output:

IDN    : ATEK1601

Band   : 3 -> (4500, 9100) MHz

10 GHz : band 4 -> (7100, 12300) MHz