ATEK1801

Programming Supplement

20-550 MHz 32-State USB-Controlled Low Pass Filter

Document revision 1.1  |  Python API version 1.1.0

This supplement contains the ATEK1801-specific programming information. It is used together with the ATEK RF Modules Programming Manual, which describes the connection, the command set, state reporting and the Python API common to all modules.

Software and example programs: https://github.com/atek-midas/atek-rf-modules

# 1  Module Identification

| Item | Value |
| --- | --- |
| Module name (*IDN? response) | ATEK1801 |
| RF chip | ATEK888BP5 |
| Description | 20-550 MHz 32-State USB-Controlled Low Pass Filter |
| States | 32 (state 0 … 31) |
| Python class | atek.TunableLowPassFilter |

RF performance data is found in the module datasheet: https://atekmidas.com/products/modules/atek1801/

# 2  States

32-band tunable low pass filter. Band n is selected with state n − 1. The front-panel buttons step through the same states.

32 bands. Nominal cut-off frequencies:

| Band | State | Cut-off (MHz) | Band | State | Cut-off (MHz) |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 27 | 17 | 16 | 150 |
| 2 | 1 | 28 | 18 | 17 | 155 |
| 3 | 2 | 29 | 19 | 18 | 160 |
| 4 | 3 | 30 | 20 | 19 | 164 |
| 5 | 4 | 31 | 21 | 20 | 168 |
| 6 | 5 | 32 | 22 | 21 | 172 |
| 7 | 6 | 33 | 23 | 22 | 176 |
| 8 | 7 | 34 | 24 | 23 | 180 |
| 9 | 8 | 48 | 25 | 24 | 230 |
| 10 | 9 | 51 | 26 | 25 | 245 |
| 11 | 10 | 55 | 27 | 26 | 265 |
| 12 | 11 | 58 | 28 | 27 | 285 |
| 13 | 12 | 68 | 29 | 28 | 310 |
| 14 | 13 | 75 | 30 | 29 | 350 |
| 15 | 14 | 95 | 31 | 30 | 425 |
| 16 | 15 | 112 | 32 | 31 | 550 |

# 3  Raw Command Example

| Direction | Line | Comment |
| --- | --- | --- |
| PC → Module | *IDN? | Identify the module |
| Module → PC | ATEK1801 |  |
| PC → Module | SET:15 | Select band 16 |
| Module → PC | STATE:15 | Confirmation |
| PC → Module | GET? | Read the current state |
| Module → PC | STATE:15 | Band 16, cut-off 112 MHz |

Command syntax and state reporting are described in Section 3 of the ATEK RF Modules Programming Manual.

# 4  Python API

## 4.1  Class atek.TunableLowPassFilter

atek.connect() returns an atek.TunableLowPassFilter object when a ATEK1801 is connected. In addition to the common methods (ATEK RF Modules Programming Manual, Section 5.5) it provides:

| Method / attribute | Description |
| --- | --- |
| set_band(band, verify=True) | Selects band 1…32. Returns the band. |
| get_band() | Returns the active band (1…32). |
| get_cutoff_mhz() | Returns the nominal cut-off frequency of the active band in MHz. |
| set_cutoff_mhz(mhz, verify=True) | Selects the lowest band whose cut-off is ≥ mhz, i.e. the tightest filter that still passes mhz. Returns the selected cut-off in MHz. |
| CUTOFFS_MHZ | Tuple of the 32 nominal cut-off frequencies, index = state. |

Band numbers start at 1; state indexes start at 0. Frequencies are given in MHz.

## 4.2  Example program

import atek_rf_modules_scpi_api as atek

with atek.connect("COM5") as lpf:              # returns an atek.TunableLowPassFilter object

print("IDN     :", lpf.identify())           # ATEK1801

lpf.set_band(10)                              # band 10 (state 9)

print("Band    :", lpf.get_band(), "->", lpf.get_cutoff_mhz(), "MHz")

fc = lpf.set_cutoff_mhz(100)                  # lowest cut-off >= 100 MHz

print("100 MHz :", "band", lpf.get_band(), "->", fc, "MHz")

Output:

IDN     : ATEK1801

Band    : 10 -> 51 MHz

100 MHz : band 16 -> 112 MHz